Before installing the application S/W VIDCAP please read the file.

1. Capture Driver Installation:

The S/W can be installed only under windows 95 OS. When PC is running in

WIN95 environment, Locate the capture driver directory(D:\win95\capture) and

run the SETUP.EXE program in MY COMPUTER.

(If CD-ROM drive letter is D:, please go to the directory D:\win95\capture first.)

Then the capture driver will be installed automatically and ask you to reboot the

system.

After the system is rebooted, then install the video capture application.

2. Use the software: (You need the package Video for Windows 1.1 for following installation)

During system is being booting in BIOS POST stage, please make sure the

setting of BIOS for Video input source should be TV-VIDEO in BIOS setup.

a. Copy the VIDCAP.EXE file to the C:\VIDCAP directory.

b. After system entering WINDOWS-95, Install use the command RUN in windows STARTUP.

the command content is as follows:

C:\VIDCAP\VIDCAP.EXE

then the information of video input signal can be viewed on screen. If the

screen doesn't have input signal, please check the VIDEO SOURCE must be

COMPOSITE1 and set Screen Overlap mode.

Note: the file NMGVCAP.INI should have the content as:

;-----------------------------------------------------------------

;

;  NeoMagic Corporation - NMG3 and NMG4 Video Capture Driver INI

;			  file for Philip's SAA7111A Video Decoder

;

;-----------------------------------------------------------------

[Hardware]

;EnableI2c=0xAddr Bit# 0|1

;EnableI2cIndexed=0xIndexAddr 0xDataAddr 0xRegIndex Bit# 0|1

[Decoder]

Address=0x48

VideoSource0=2

VideoSource1=3

VideoSource2=7

VideoSource=0

VideoStd=0

VideoSubStd=0

Brightness=128

Contrast=71

Saturation=64

Hue=0

Sharpness=0

[VideoCapture]

VideoWidth=352

VideoHeight=240

VideoFormat=1